------S1------

hostname S1

ip domain-name halasz.noel

int range fa0/1-2

desc LANA

switchport mode access

switchport access vlan 10

exit

int fa0/3

desc SRV

switchport mode access

int fa0/24

desc TRUNK-S1-S2

switchport mode trunk

switchport trunk allowed vlan 1,10

int vlan 1

desc Mngmnt

ip address 192.168.0.2 255.255.255.248

no sh

exit

line vty 0 4

login

exit

line vty 0 15

login

exit

-----S2-----

hostname S2

ip dhcp pool LANA

network 192.168.0.16 255.255.255.240

default-router 192.168.0.30

exit

ip routing

int range fa0/1-2

desc LANB

switchport access vlan 20

switchport mode access

switchport nonegotiate

exit

fa0/23

description Mngmnt

switchport mode access

switchport nonegotiate

exit

fa0/24

switchport mode trunk

switchport trunk allowed vlan 1,10,20

switchport trunk encapsulation dot1q

description TRUNK-S1-S2

exit

int vlan 10

description LANA

mac-address 0001.63aa.1901

ip address 192.168.0.30 255.255.255.240

exit

int vlan 20

description LANB

ip address 192.168.0.46 255.255.255.240

ip helper-address 192.168.0.1

exit

int vlan 1

ip address 192.168.0.3 255.255.255.248

no sh

exit

-----R1-----

hostname R1

int fa0/0

ip address 192.168.0.6 255.255.255.248

no sh

exit

ip route 192.168.0.16 255.255.255.240 192.168.0.3

ip route 192.168.0.32 255.255.255.240 192.168.0.3

ip dhcp excluded-address 192.168.0.1

ip dhcp excluded-address 192.168.0.2

ip dhcp excluded-address 192.168.0.4 192.168.0.8

ip dhcp pool Mngmnt

network 192.168.0.0 255.255.255.248

default-router 192.168.0.6

exit

enable password cisco

username admin password cisco123

line vty 0 1

login local

line vty 2 4

login